## time_features

## Fungsi Utama

Menangkap karakter sinyal di domain waktu
→ impact, impulsiveness, noise, shock

📌 Dipakai untuk:

- early fault detection

- anomaly ML

- TIDAK untuk operator display

Penjelasan Fungsi

| Feature | Fungsi | Catatan |
| --- | --- | --- |
| acc_rms | Energi total | Backend / ML |
| acc_peak | Shock | Jangan alarm sendiri |
| crest_factor | Impact / spall | Lebih stabil dari peak |
| kurtosis | Impulsiveness | Kalah stabil dari envelope |
| skewness | Asimetri | ML only |

## frequency_features

## 🎯 Fungsi Utama

Menangkap energi per band frekuensi
→ lokasi fault (low / mid / high)

📌 Dipakai untuk:

- energy ratio

- trend severity

- bearing early fault

## 🧠 Penjelasan Fungsi

| Band | Makna Fisik |
| --- | --- |
| LF | unbalance, misalignment |
| MF | looseness, gear wear |
| HF | bearing, cavitation |

📌 Display?

- Operator: ❌

- Maintenance: ❌ (pakai energy_ratio saja)

- ML: ✅

## order_features

## 🎯 Fungsi Utama

Menangkap harmonic berbasis RPM (order)
→ identifikasi jenis fault

📌 Dipakai untuk:

- fault classification

- engineering confirmation

## Penjelasan Fungsi

| Order | Indikasi |
| --- | --- |
| 1× | unbalance |
| 2× | misalignment |
| 3× | looseness |

📌 Display?

- Operator: ❌

- Maintenance: ❌

- Engineering: ✅ (optional)

RINGKASAN: FEATURE → FUNGSI → DISPLAY

| ## Feature Group | ## Fungsi | ## Display |
| --- | --- | --- |
| ## time_features | ## impact / impulse | ## ML only |
| ## frequency_features | ## energy distribution | ## ML only |
| ## envelope | ## bearing early fault | ## Maintenance |
| ## energy_ratio | ## severity bearing | ## Maintenance |
| ## order_features | ## fault type | ## Engineering |
| ## health_index | ## decision | ## Operator |